TASK 1 . Create VLANs: Sales and Support

Use one 2960 switch in Cisco Packet Tracer.

VLAN configuration

Open the switch CLI and enter:

enable

configure terminal

vlan 10

name Sales

exit

vlan 20

name Support

exit

This creates:

- VLAN 10 → Sales

- VLAN 20 → Support

2. Assign Switch Ports and Verify

Let's assign three ports to Sales and two ports to Support.

Sales — VLAN 10

We'll use FastEthernet 0/1 through 0/3:

interface range fastEthernet 0/1-3

switchport mode access

switchport access vlan 10

exit

Support — VLAN 20

We'll use FastEthernet 0/4 and 0/5:

interface range fastEthernet 0/4-5

switchport mode access

switchport access vlan 20

exit

Verify VLAN membership

Run:

show vlan brief

You should see something similar to:

VLAN Name                             Status    Ports

1    default                          active    Fa0/6, Fa0/7, ...

10   Sales                            active    Fa0/1, Fa0/2, Fa0/3

20   Support                          active    Fa0/4, Fa0/5

Expected result

| VLAN ID | VLAN Name | Ports |
| --- | --- | --- |
| 10 | Sales | Fa0/1, Fa0/2, Fa0/3 |
| 20 | Support | Fa0/4, Fa0/5 |

3. Connect Two Routers and Configure a Static Route

For a simple Packet Tracer setup, use an Ethernet connection between the routers.

Example topology

LAN 1                         LAN 2

192.168.1.0/24                192.168.2.0/24

│                              │

│                              │

┌───▼───┐                      ┌───▼───┐

│Router 1│────── Ethernet ─────│Router 2│

└───────┘                      └───────┘

G0/0:                           G0/0:

192.168.1.1                    192.168.2.1

│

R1 ↔ R2 link

10.0.0.1/30

10.0.0.2/30

Router 1

enable

configure terminal

interface gigabitEthernet 0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface gigabitEthernet 0/1

ip address 10.0.0.1 255.255.255.252

no shutdown

exit

Router 2

enable

configure terminal

interface gigabitEthernet 0/0

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

interface gigabitEthernet 0/1

ip address 10.0.0.2 255.255.255.252

no shutdown

exit

Static route on Router 1

Router 1 needs to know how to reach Router 2's LAN:

ip route 192.168.2.0 255.255.255.0 10.0.0.2

This means:

"To reach the 192.168.2.0/24 network, send the packets to Router 2 at 10.0.0.2."

You can verify the route with:

show ip route

4. Sales VLAN → Support VLAN Ping

Suppose:

- Sales PC = 192.168.10.10, VLAN 10

- Support PC = 192.168.20.10, VLAN 20

If the Sales PC runs:

ping 192.168.20.10

the ping will fail if the network only contains the Layer 2 switch configured with VLAN 10 and VLAN 20.

Why does it fail?

VLANs create separate broadcast domains. A Layer 2 switch does not normally route traffic between VLAN 10 and VLAN 20.

Sales VLAN 10                 Support VLAN 20

┌─────────┐                   ┌─────────┐

│ Sales PC│                   │Support PC│

└────┬────┘                   └────┬────┘

│                             │

│                             │

┌───▼─────────────────────────────▼───┐

│              SWITCH                  │

│                                      │

│ VLAN 10  ║  VLAN 20                 │

│           ║                          │

└───────────╫──────────────────────────┘

║

❌ No routing

The switch keeps the VLANs logically separated, so the Sales device cannot directly communicate with the Support device.

How to Allow Inter-VLAN Communication

One solution is Router-on-a-Stick.

The router uses separate subinterfaces, one for each VLAN:

Router

┌──────────────┐

│              │

G0/0.10            G0/0.20

VLAN 10             VLAN 20

192.168.10.1        192.168.20.1

│              │

└──────┬───────┘

│

Trunk Link

│

┌─────▼─────┐

│  Switch   │

└─────┬─────┘

╱       ╲

╱         ╲

Sales PC     Support PC

VLAN 10       VLAN 20

The router can then route packets between VLAN 10 and VLAN 20.

Another option is to use a Layer 3 switch and configure switched virtual interfaces (SVIs) for the VLANs.